| FEATURES: Three Phase Silent Type Diesel Generator |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Oil Level Alarm When the oil level is low that the engine will not start. |  |  | Circuit Protector Circuit Protector is a device capable of carrying and interrupting both load and fault current up to a certain rating |
|  | AVR AVR is a device often solid state, for controlling the output voltage of a generator |  |  | Large Fuel Tank Capacity |
|  | Digital Control Panel Digital control panel (V, A, Hz, kW, Bat V values are displayed on the control panel). |  |  | Battery 12 V – 36 A |
|  | Sound Insulated Protection Cabins Its designed to provide ideal sound level & protection. |  |  | Wheel Transport System |
|  |  |  |  | ATS Socket |

| Generator |  | Engine |  | Size |  |
| --- | --- | --- | --- | --- | --- |
| Standby Power | 10 kVA | Model | 195FE | Weight | 185/190 kg |
| Prime Power | 9 kVA | Max. Output Power | 13 hp / 9.6 kW | Width | 1120 mm |
| Continues Rated Current | 38.88 A | Rotation Speed | 3000 / 3600 r/min | Length | 545 mm |
| Dc Output | 12V 8.3A | Cooling System | Air-Cooled | Height | 705 mm |
| Starting System | Electrical Start | Excitation Mode | SCR Self-Excited |  |  |
| Rated Voltage | 220 / 380 V | Oil Type | 10-30 or 15-40 |  |  |
| Fuel Type | Diesel | Fuel Tank Capacity | 16 L |  |  |
| Number of Phases: | Single / Three Phase | Bore × Stroke | 95 x 75 mm |  |  |
| Noise Level | 78 Db | Displacement | 0.535 L |  |  |
| Continuous Operating Time | 12 H | Lubrication oil capacity | 1.65 L |  |  |
| Rated Frequency | 50 / 60 Hz | Engine Type | 1 Cylinder, 4-Stroke, Direct Injection, Diesel Engine |  |  |
| Power Factor (cosφ) | 1.0 / 0.8 |  |  |  |  |

Continuous Power

The maximum power which a generating set is capable of delivering continuouslywhilst supplying a constant electrical load. Average load can be 100%. The generator must not be overloaded.

Standby Power
The max. power available during a variable electrical power sequence, under the stated operating conditions, for which a generating set is capable of delivering in the event of a utilitypower outage or under test conditions for up to 200 hrs of operation per year under average of 70%load.Overloading isn’t permissible.

Prime Power

The maximum power which a generating set is capable of delivering continuouslywhilst supplying a variable electrical load. Average load should be 70%. The generator can be overloaded 10% for 1 hour per 12 hrs.