| FEATURES: Single Phase Open Frame Double Cylinders Gasoline Generator, Powered by NSM Alternator Ideal for Charging T-40 and T-30 Agricultural Spraying Drones |  |  |  |
| --- | --- | --- | --- |
|  | OHV Over Head Valve Easy To Maintenance |  | Single Plug Possible To Getting A Full Load From A Single Plug (63 A) |
|  | AVR AVR Is a Device Often Solid State, For Controlling The Output Voltage of a Generator |  | Choke System When The Cold Weather, Choke System Aid To Operate The Engine. |
|  | Wheel Transport System |  | Dual Element Air Filter Dual-Element Air Filter That Purifies The Sucked Air From Dust And Dirt |

| Generator |  | Engine |  | Size |  |
| --- | --- | --- | --- | --- | --- |
| Standby Power | 16.5 kVA | Model | QST2V80FE | Weight | 150 kg |
| Prime Power | 15 kVA | Max. Output Power | 28 hp / 20.7 kw | Width | 600 mm |
| Continues Rated Current | 65.25 A | Rotation Speed | 3000 / 3600 r/min | Length | 900 mm |
| Dc Output | 8.3 A / 12v | Cooling System | Air-Cooled | Height | 900 mm |
| Starting System | Electrical Start | Ignition System | TCI |  |  |
| Rated Voltage | 220 V | Oil Type | 10-30 or 15-40 |  |  |
| Fuel Type | Gasoline | Fuel Tank Capacity | 20 L |  |  |
| Number of Phases | Single Phase | Bore×Stroke | 80 x 74 mm |  |  |
| Noise Level | 74 Db | Displacement | 743 mm |  |  |
| Continuous Operating Time | 12 H | Lubrication oil capacity | 1.7 L |  |  |
| Rated Frequency | 50 Hz | Engine Type | 2 Cylinder, 4Stroke, OHV, Gasoline Engine |  |  |
| Power Factor (cosφ) | 1.0 / 0.8 |  |  |  |  |

Continuous Power

The maximum power which a generating set is capable of delivering continuouslywhilst supplying a constant electrical load. Average load can be 100%. The generator must not be overloaded.

Standby Power
The maxpower available during a variable electrical power sequence, under the stated operating conditions, for which a generating set is capable of delivering in the event of a utilitypower outage or under test conditions for up to 200 hrs of operation per year under average of 70%load.Overloading isn’t permissible.

Prime Power

The maximum power which a generating set is capable of delivering continuouslywhilst supplying a variable electrical load. Average load should be 70%. The generator can be overloaded 10% for 1 hour per 12 hrs.